APPENDIX 4 - Sail Use and Measurement

Section A – Conditions for Racing

A.1	SAILS

A.1.1	MODIFICATIONS, MAINTENANCE AND REPAIR

(a)	After certification, sails shall not be altered in any way except as permitted by these class rules.

(b) The following is permitted without re-certification or approval and may be done by anyone:

(1)	Routine maintenance

(2)	Addition of tell tales

(3)	Addition of camber stripes

(4)	Battens may be placed in the batten pockets

(5)	Minor repairs, such as applying tape to holes or tears in the body of the sail.

A.1.2	LIMITATIONS

(a)	Not more than one mainsail, one headsail and one spinnaker shall be carried aboard.

(b)	Not more than two mainsails, two headsails and two spinnakers shall be used during a World or a Continental Championship, except when a sail has been lost or damaged beyond repair.

A.1.3	mainSAIL

(a)	USE

(1)	No part of the mainsail shall extend above the lower edge of the mast upper limit mark.

(2)	No part of the mainsail shall extend behind the fore side of the boom outer limit mark.

(3)	Luff and foot bolt ropes shall be in the spar grooves.

Section B – Sails

B.1	Parts

B.1.1	Mandatory

(a)	Mainsail

(b)	Headsail

B.1.2	Optional

(a)	Spinnaker

B.2	General

B.2.1	Rules

(a)	Sails shall comply with the class rules in force at the time of certification.

B.2.2	Certification

(a)	The official measurer shall certify mainsails and headsails in the tack and spinnakers in the head and shall sign and date the certification mark.

(b)	An ISAF In-House Certification (IHC) Authorizing Authority may appoint one or more In-House Official Measurer(s) to measure and certify sails produced by a manufacturer.

B.2.3	Sailmaker

(a)	No licence is required.

B.2.4	MODIFICATIONS, MAINTENANCE AND REPAIR

(a)	Alterations as described in Section A are permitted but any additional work requires new certification control and re-certification.

B.3	Mainsail

B.3.1	Identification

(a)	The International Fireball Class insignia shall be a red disc, placed back to back on the two sides of the mainsail.

| Measurement | minimum | maximum |
| --- | --- | --- |
| Class insignia diameter | 497 mm | 523 mm |

(b)	National letters must be in accordance with ISAF RRS Appendix G. ISAF RRS Appendix G1.1(b) is modified such that the mainsail shall carry national letters.

(c)	The sail number must be in accordance with ISAF RRS Appendix G. The sail number is allotted by the class association.

B.3.2	Materials

(a)	The body of the sail shall consist of woven ply or laminated ply.

(b)	Where the body of the sail is made of laminated ply, the film shall be of polyester and the ply fibres shall be of polyester, polyethylene or aramid.

(c)	Where the body of the sail is made of woven ply, the ply fibres shall be of polyester.

(d)	Primary reinforcement and secondary reinforcement may be of woven ply or laminated ply.

(e)	Battens shall be made of glass fibre.

B.3.3	Construction

(a)	The construction shall be: soft sail, single-ply sail.

(b)	The body of the sail shall consist of the same woven ply or laminated ply throughout, except that within 400 mm of the foot an alternate material is permitted.

(c)	The sail shall have four batten pockets in the leech.

(d)	The sail may be constructed so that it can be reefed by means of one point adjacent to the leech no more than 275 mm from the clew point. No other reefing points are permitted.

| Measurement | minimum | maximum |
| --- | --- | --- |
| Reefing point to clew point | - | 275 mm |

(e)	Only the following are permitted: Seams, stitching, glues, tapes, primary reinforcement, secondary reinforcement, tabling, bolt ropes, corner eyes or webbing, headboard with fixings, Cunningham eye or pulley, batten pockets,  batten pocket patches, batten pocket elastic, uppermost batten pocket end cap at luff end and tensioning or fastening device at leech end, leech line with cleat on leech, not more than two windows, one boom slide fixed at the clew, tell tales, sail shape indicator stripes, sail identification, sailmaker labels, certification mark, and items as permitted or prescribed by other applicable rules.

(f)	The leech shall not extend aft of straight lines between:

(1)	the aft head point and the intersection of the leech and the upper edge of the nearest batten pocket,

(2)	the intersection of the leech and the lower edge of a batten pocket and the intersection of the leech and the upper edge of an adjacent batten pocket below,

(3)	the clew point and the intersection of the leech and the lower edge of the nearest batten pocket.

(g)	The sail shall not be a double luff sail.

B.3.4	Dimensions

| Measurement | minimum | maximum |
| --- | --- | --- |
| Leech length | - | 6350 mm |
| Quarter width | - | 2550 mm |
| Half width | - | 1940 mm |
| Three-quarter width | - | 1140 mm |
| Top width | - | 180 mm |
| Primary reinforcement | - | 320 mm |
| Secondary reinforcement: |  |  |
| from sail corner measurement points | - | 960 mm |
| from Cunningham eye or pulley | - | 960 mm |
| for flutter patches | - | 105 mm |
| for chafing patches | - | 960 mm |
| for batten pocket patches | - | 320 mm |
| at a reefing point adjacent to leech | - | 960 mm |
| Tabling width | - | 100 mm |
| Distance from clew point to foot bolt rope | - | 100 mm |
| Distance from tack point to foot bolt rope | - | 450 mm |
| Total window(s) area | - | 0.28 m2 |
| Window(s) to sail edges | 150 mm | - |
| Vertical extension of headboard from head point | - | 160 mm |
| Batten pocket length: |  |  |
| uppermost pocket: inside | - | 1170 mm |
| intermediate pockets: inside | - | 885 mm |
| lowermost pocket: inside | - | 735 mm |
| Batten pocket width: |  |  |
| inside | - | 50 mm |
| outside | - | 80 mm |
| Head point to intersection of leech and centreline of uppermost batten pocket | 1285 mm | - |
| Clew point to intersection of leech and centreline of lowermost batten pocket | 800 mm | - |
| Distance from lowermost batten pocket to foot | 800 mm | - |

B.4	Headsail

B.4.1	Materials

(a)	The body of the sail shall consist of woven ply or laminated ply.

(b)	Where the body of the sail is made of laminated ply, the film shall be of polyester and the ply fibres shall be of polyester, polyethylene or aramid.

(c)	Where the body of the sail is made of woven ply, the ply fibres shall be of polyester.

(d)	Primary reinforcement and secondary reinforcement may be of woven ply or laminated ply.

B.4.2	Construction

(a)	The construction shall be: soft sail, single-ply sail.

(b)	The body of the sail shall consist of the same woven ply or laminated ply throughout.

(c)	The leech shall not extend beyond a straight line from the aft head point to the clew point.

(d)	Only the following are permitted: Seams, stitching, glues, tapes, primary reinforcement, secondary reinforcement, tabling, corner eyes or webbing, Cunningham eye, hanks, leech line with cleat on leech, not more than two windows, tell tales, sail shape indicator stripes, sail identification, sailmaker labels, certification mark, and items as permitted or prescribed by other applicable rules.

(e)	The luff shall not enclose the forestay.

B.4.3	Dimensions

| Measurement | minimum | maximum |
| --- | --- | --- |
| Luff length | - | 4320 mm |
| Luff Perpendicular | - | 1525 mm |
| Foot median minus the mean of the luff length and the leech length | - | 155 mm |
| Top width | - | 40 mm |
| Primary reinforcement | - | 280 mm |
| Secondary reinforcement: |  |  |
| from sail corner measurement points | - | 840 mm |
| for flutter patches | - | 75 mm |
| for chafing patches | - | 840 mm |
| Tabling width | - | 40 mm |
| Total Window(s) area | - | 0.28 m2 |
| Window(s) to sail edges | 150 mm | - |

B.5	Spinnaker

B.5.1	IDENTIFICATION

(a)	The sail number must be in accordance with ISAF RRS Appendix G. The sail number shall be the same as that on the mainsail.

(b) 	ISAF RRS Appendix G.1.3(d) is modified such that national letters are not required on spinnakers.

B.5.2	Materials

(a)	The body of the sail, primary reinforcement and secondary reinforcement shall be of nylon or polyester woven ply.

B.5.3	Construction

(a)	The construction shall be: soft sail, single-ply sail.

(b)	The body of the sail shall consist of the same woven ply throughout. This limitation does not apply to the colour of the ply, which may differ from panel to panel.

(c)	Only the following are permitted: Seams, stitching, glues, tapes, tabling, primary reinforcement, secondary reinforcement, corner eyes or webbing, recovery line eyes or webbing, tell tales, sail indicator stripes, sail identification, sailmaker labels, certification mark, and items as permitted or prescribed by other applicable rules.

B.5.4	Dimensions

| Measurement | minimum | maximum |
| --- | --- | --- |
| Leech length and luff length | - | 4350 mm |
| Foot length | - | 3000 mm |
| Foot Median | - | 5100 mm |
| Difference between diagonals | - | 50 mm |
| Half width | 2810 mm | 3450 mm |
| Primary reinforcement | - | 280 mm |
| Secondary reinforcement: |  |  |
| from sail corner measurement points | - | 840 mm |
| For flutter patches | - | 75 mm |
| for recovery line point | - | 320 mm |
| Tabling width | - | 40 mm |